SECTION 202

REMOVAL OF STRUCTURES AND OBSTRUCTIONS

Special Provisions

202-1.01 DESCRIPTION.  Add the following:

This work shall also include the removal of dowelled curbs and the repair of the cavities left after the removal.

This work shall also include the capping, grouting, and abandoning culvert pipes shown in the plans.

51132-081910

This work shall also include the removal of gas pipe, removal of storage tanks (either fuel or septic) and the decommissioning of ground water wells.

CR112-010610

202-3.01 GENERAL.  Add the following:

The removal of buildings, foundations, and structures shall include removal of utility lines, sidewalks, and other attached appurtenances.

Buildings being removed may contain asbestos.  Take special handling measures when removing this material.

Cut the ends of retaining walls to remain after partial removal neat and true with no shatter outside the removal area.

Remove septic or fuel tanks according to the Department of Environmental Conservation (DEC) regulation.  Remove or decommission wells according to DEC regulations.

CR112-010610

Carefully remove fences designated by the Engineer to the right-of-way limit, or to the end of the span beyond the right-of-way limit.  These materials belong to the property owners, and shall be salvaged and stacked neatly in their yards.  After the construction of noise barrier or fence is complete, use salvaged fencing to fill possible fencing gaps behind the property line.  Use salvaged fencing according to Section 607, for reconstructed fences.

CR17-102402

For purposes of this Contract, “other obstructions” may include abandoned items within the Right-of-Way and may include appliances, vehicles, tanks, tools, equipment, and other such considered junk items.

Fill the cavities left after the removal of dowelled curb in a manner acceptable to the Engineer.

202-3.02 MAIL BOXES.  Add the following:

Contact Dan Carper (USPS Postmaster, 745-5051) one week in advance of mail box removing or resetting so he can coordinate the locations and notify affected property owners.

202-3.03 REMOVAL OF BRIDGES, CULVERTS, AND OTHER DRAINAGE STRUCTURES.  Replace the third paragraph with the following:

When culvert pipe is designated on the Plans to be abandoned in place, grout the entirety of the pipe from end to end by an approved method. Securely plug other conduits by an approved method.

51132-033011

202-3.04 REMOVAL OF PIPE.  Add the following:

During project construction, ENSTAR Natural Gas Company will abandon existing gas pipelines.  Follow the procedures given in Subsection 105-1.06 Cooperation With Utilities, when working around gas pipelines until they have been abandoned.  Once the pipelines are abandoned, remove abandoned natural gas pipe within the right-of-way that are in conflict with the work and dispose of pipes off the project limits.

CR112-010610

202-3.05 REMOVAL OF PAVEMENT, SIDEWALKS, AND CURBS.  Add the following:

Removed pavement material, including sidewalks and curbs, is the property of the Contractor.  Handle and transport materials according to the Alaska Department of Environmental Conservation (DEC) regulations.  Store materials at the Contractor’s DEC approved site.

Removed pavement, sidewalks, and curbs may be used for embankment construction if it is not exposed at the completed embankment surface.  Maximum allowable dimension of broken materials is 6 inches.  The use of pavement, sidewalk, and curb in the embankment requires written approval and direction for use from the Engineer.

CR2022-010610

Dowels from curb may not be placed in the embankment.

Dowelled curb shall be removed in a manner that leaves the underlying pavement intact. Damage done to the pavement during the removal of the dowelled curb shall be repaired by the Contractor, in a manner acceptable to the Engineer, at no additional cost to the Department. Remove the existing aggregate base course placed on the asphalt between the existing curb and gutter and dowelled curb from Station 156+25 to Station 163+75. Wash and/or sweep the paved surface clean and free of loose materials to the satisfaction of the Engineer.

51132-081910

Dispose of removed pavement, sidewalks, and curbs not wanted by the Contractor and not used in the project, according to subsection 202-3.09.

CR2022-010610

Add the following Subsection 3.06:

202-3.06 - RESERVED

51132-032211

Add the following Subsection 3.07:

202-3.07 GROUND WATER WELL DECOMMISSIONING.  Decommission the water wells according to DEC requirements conforming to 18AAC 80.015 Well Protection Source Water Protection, and Well Decommissioning, or a DEC approved alternate method.  Develop and submit a detailed Ground Water Well Decommissioning Plan to the Engineer.  Contact the Wasilla DEC office at (907) 376-5038, for plan review requirements of alternate methods.

CR112-010610

Add the following Subsection 3.08:

202-3.08 - RESERVED

51132-032211

Add the following Subsection 3.09.

202-3.09 DISPOSAL OF PAVEMENT, SIDEWALKS, AND CURBS.

Disposal sites shall be outside the project limits unless directed otherwise, in writing, by the Engineer.  Obtain written consent from the property owner.  Dispose of solid waste materials, pavement, sidewalk, and curb (including handling, transporting, storing and disposing) according to the Alaska Department of Environmental Conservation (DEC) Regulations.

A DEC Permitting Officer in Anchorage may be contacted at (907) 269-7590.

CR2021-010610

202-5.01 BASIS OF PAYMENT.  Add the following:

Acquiring a solid waste disposal permit from DEC is subsidiary to 202 Pay Items.

CR2021-010610

Item 202(1).  Also includes removing and disposing all “other obstructions” (aka junk items) within the Right-of-Way.

Add the following to the end of the second paragraph

Removal of dowelled curb shall be paid for as item 202(9) Removal of Curb and Gutter. Payment includes full compensation for labor, materials, and equipment for the removal, disposal, repair of cavities, removal of aggregate base course on the existing pavement, and cleaning of the paved surface. Grouting and abandoning of culvert pipes is subsidiary to 202(4) Removal of Culvert Pipe.

51132-080510

Add the following:

Item 202(13).  At the Contract Unit price for the actual length of fence taken down, disposed or delivered to the owner, regardless of the type or height.  Payment includes full compensation for labor and materials required to perform this work.

CR17-102402

Item 202(16).  At the Contract Unit price for the length of pipe removed, regardless of size.  Payment includes full compensation for labor, materials, and equipment required to remove and dispose of the pipes as shown in the Plans.

Item 202(18).  At the Contract Lump Sum price for each tank removed and disposed of according to DEC regulations.  Payment includes full compensation for labor, materials, necessary excavation, purging, processing, or removing fuel contaminated soils around the tanks, and backfilling the resultant hole.

Item 202(23A).  At the Contract Unit price for each well decommissioned according to DEC regulations.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

202(13)	Removal of Fence	Linear Foot

CR17-102402

Pay Item No.	Pay Item	Pay Unit

202(16)	Removal of Gas Pipe	Linear Foot

202(18)	Removal of Storage Tanks	Each

202(23A)	Ground Water Well Decommissioning	Each

CR112-010610