- SECTION 615

STANDARD SIGNS

Add the following subsection:

615-3.03 ISLAND DELINEATORS. Install island delineators at the locations shown on the plans. Each island delineators shall be a two part system: a base and a post.

The base shall be galvanized metal designed to be driven into dense soil, or cast into concrete. When installed, the base shall not extend more than 2 inches above the surrounding finished grade. The base shall be designed to allow installation and replacement of the post without the use of tools.

The post shall be a flexible polymer tube that is resistant to ultraviolet light, hydrocarbon damage, and impact resistant between -20ºF and 120ºF. The post shall have a diameter of 2.25 inches and shall extend 48 inches above the top of the base when installed. The post shall have a minimum strength of 1800 pounds per square inch, and be capable of withstanding a minimum of 10 vehicle bumper impacts at 55 mph without fracturing.

Provide orange posts with white reflective sheeting on the side facing traffic. Install delineator bases to ensure reflective sheeting faces oncoming traffic.

51918-032911

615-5.01 BASIS OF PAYMENT. Replace the first sentence with the following: Sign posts, bases, mounting hardware, and concrete used for sign bases are subsidiary to 615 Pay Items.

51927/51918-110410

Add the following:

Island Delineators shall be subsidiary to Pay Item 615(1)

51918-032911