SECTION 619

SOIL STABILIZATION

Special Provisions

619-1.01 DESCRIPTION.  Add the following:  This work also includes furnishing and installing of cellular slope protection, where shown on the Plans or as directed by the Engineer.

619-1.02 SUBMITTALS. Add the following:

Submit a Manufacturer’s Certificate of compliance as defined in subsection 106-1.05, Certification of Compliance, for materials to be placed under Item 619(6) Cellular Slope Protection.  Include the manufacturer’s installation instructions for the application shown on the Plans.

619-2.01 MATERIALS.  Replace this subsection with the following:

Mulch	subsection 727-2.01

Matting	subsection 727-2.02, item 4. Knitted Straw mat

Staples	subsection 727-2.03

Topsoil	Section 726

Cellular Slope Protection Materials

1.	General. Use cellular slope protection systems pre-manufactured as an assembly of expandable connected cells constructed of high-density polyethylene (HDPE).  Use non-perforated cell walls.  Firmly anchor the slope protection and backfill as recommended by the manufacturer.

2.	Cellular Slope Protection System. Use one of the following cellular slope protection systems or an approved equivalent:

a.	“Geoweb 4” Standard Cell”, as manufactured by Presto Products Company, P.O. Box 2399 Appleton, Wisconsin, 54913, (800) 548-3424

b.	“EnviroGrid 4”, as manufactured by AGH Industries, Inc., P.O. Box 1852, Fort Worth, Texas, 76101, (817) 284-1742

- “GeoLok 400”, as manufactured by Fluid Systems, Inc., 1245 Corporate Boulevard, Suite 300, Aurora, Illinois, 60504, (800) 346-9107

3.	Prefabricated Slope Protection System Material: Shall meet the following requirements:

Nominal Cell Depth (min.)	4 inches

Sheet Thickness (min.)	50 mil ± 5 mil

Tensile Seam Strength (min.)	225 lbf

Standard Sheet Configuration (typ.)	8 feet x 20 feet

Individual Expanded Cell (max.)	10 inches x 8 inches

4.	Cell Backfill Material:  Meet the requirements for subsection 703-2.10 Porous Backfill Material.

Add the following subsection:

619-3.04 CELLULAR SLOPE PROTECTION.  Construct the cellular slope protection according to the details on the approved working drawings and consistent with the manufacturer’s recommendations.

Foundation Preparation.  Construct slopes to the lines and grades shown on the Plans before construction of the cellular slope protection.  Bench to place and compact Selected Material, Type A.  Level or fill localized high or low spots.  Remove obstructions such as projecting stones.  Remove soft or low density pockets of material and backfill with Selected Material, Type A.

Construction Requirements.  Place the cellular slope protection mats in a manner as to produce a smooth surface and construct within the specified lines and grades shown on the Plans.  Follow the manufacturer’s recommendations for construction of the cellular slope protection system.  Place topsoil and seed where shown on the Plans.

Stretch the mat to the standard nominal expanded dimensions and anchor it according to the manufacturer’s recommendations.  Place the mats side by side so that the mats interlock.  Join or anchor the mats according to the manufacturer’s recommendations.

Infill the cells according to the manufacturers recommendations.  Place topsoil and seed where shown on the Plans.

619-4.01 METHOD OF MEASUREMENT.  Add the following

Item 619(6) Cellular Slope Protection shall be measured by the square yard on the ground surface.

619-5.01 BASIS OF PAYMENT.  Add the following pay items:

Item 619(6) Cellular Slope Protection includes furnishing cellular slope protection mats and associated hardware, and placing, staking, and backfilling the mats with cell backfill material.  Foundation preparation before installing cellular slope protection, including grading, excavation, backfilling with Selected Material, Type A, and topsoil will be subsidiary to Item 619(6) and no separate payment will be made.  Riprap and geotextile will be paid under Sections 611 and 631 respectively.

Pay Item No.	Pay Item	Pay Unit

619(6)	Cellular Slope Protection	Square Yard