SECTION 501

STRUCTURAL CONCRETE

Special Provisions

501-1.01 DESCRIPTION.  Add the following:  Drill into portions of the existing bridge as described in this Contract.  Remove and replace damaged portions of concrete of the existing bridge as described in this Contract.

501-3.08 PLACING CONCRETE.  Add the following:

15.	Drilling and Bonding.  Drilling of concrete consists of various diameters through reinforced and prestressed concrete bridge members, as shown on the plans and in conformance with the requirements in these special provisions.  Drill holes by methods that do not shatter or damage the concrete adjacent to the holes.

Repair all damage to existing concrete, which is to remain in place, to a condition equal to that existing prior to the beginning of the drilling operations.  The cost of repairing existing concrete damaged by the Contractor’s operation is at the Contractor’s expense.

Do not damage or otherwise contact reinforcing steel or prestressing steel when drilling through reinforced and prestressed concrete members.  Locate existing reinforcing steel and prestressing steel prior to drilling holes.  Provide results of the field survey to the Engineer prior to drilling holes.

Holes in which reinforcing or prestressing steel is encountered during drilling before the specified depth is attained will be rejected.  Drill a new hole, which does not strike reinforcing steel or prestressed steel, adjacent to the rejected hole to the specified depth.  Fill all rejected holes with epoxy grout.

501-4.01 METHOD OF MEASUREMENT.  Add the following:

Class W Concrete: measured per lump sum for all Class W concrete required for headwalls as shown on the plans.

501-5.01 BASIS OF PAYMENT.  Add the following:

All materials and labor required for installation of anchor rods are subsidiary to item 615(12) Bridge Sign Mount