SECTION 726

TOPSOIL

Special Provisions

726-2.01 TOPSOIL.  Replace item (2.) with the following:

2.	Topsoil shall contain not less than 15 percent and no greater than 25 percent organic matter by dry weight as determined by ATM T-6.

Delete the gradation table in the third paragraph and substitute the following:

Sieve Designation	Percent Passing (by Weight)

10	100

20	 90

100 	 50

Delete last sentence in second paragraph and substitute the following:

The Contractor shall submit to the Engineer, 10 working days prior to final placement, a soil analysis for particle size, nutrient content and organic content, before any topsoil shall be accepted for this project.  The Engineer may test the topsoil at any time to verify compliance with the topsoil analysis.  If the supplied topsoil does not meet specifications in this Section, the Contractor shall supply new topsoil of a quality required to meet the section standards.

Delete the fertilizer requirements (not the limestone requirements) as stated in this subsection and substitute the following:

Nitrogen	21 – 35 PPM

Phosphoric Acid	11 - 20 PPM

Potassium	76 - 150 PPM

TABLE 726-1

LIMESTONE REQUIREMENTS

Add the following:

Limestone shall be incorporated into the topsoil at a sufficient rate to attain a pH of between 6.0 and 7.0.