- SECTION 508

WATERPROOFING MEMBRANE

Special Provision

508-1.01 DESCRIPTION.  Add the following:  Comply with the Construction permit, issued by the EPA under the NPDES and Section 641 during the phase of construction including the removal of existing asphalt pavement, the waterproofing membrane, and the application of new membrane, asphalt and concrete materials.

Remove the existing asphalt pavement and membrane waterproofing on the following bridge

| Bridge Number | Bridge Name |
| --- | --- |
| 0986 | Buskin River Bridge |

508-3.01 APPLICATION OF MEMBRANE WATERPROOOFING.

- General.  Replace a, b, and c with the following:

- Complete bond between the leveling course and concrete bridge deck.

- Complete bond between the membrane and the leveling course.

- Unbroken waterproof membrane in place between the leveling course and the asphalt overlay.

Add d.

- Complete bond between the membrane and the asphalt overlay.

- Preparation of Concrete Deck.  Before applying membrane: Replace a. through f. with the following:

- Remove existing asphalt, concrete, and membrane down to the base concrete bridge deck.

- Do not employ removal methods, manual or mechanical, that damage the concrete deck.  Mechanical milling machines such as reclaimers or planers must not damage the concrete deck.

- Remove any contaminants such as grease, oil, or paint with solvents, detergents or by sand blasting as approved by the Engineer.  Do not damage the existing concrete deck and deck materials.

- Sweep the deck to remove foreign materials such as dirt, dust, moisture, and loose concrete.  Sweep and power broom according to subsection 643-3.04 Traffic Control Devices, Item 6., Street Sweeping and Power Brooming.

- Before applying the tack coat, primer, or emulsion, thoroughly clean the deck with compressed air.  Contain disturbed particles, dust, or other particles similar to street sweeping or power brooming.

- Prevent foreign material from entering streams including fugitive dust particles.

508-5.01 BASIS OF PAYMENT.  Add the following:

HMA Type II; Class A and HMA Type IV; Class B are paid for under the paving items.

Pavement planing on bridge decks is paid under item 202(15) Pavement Planing.  Additional removal of existing pavement below the pavement planing depth to bare concrete deck is subsidiary to 508 pay items.  Removal of existing waterproof membrane is subsidiary to 508 pay items.