Add the following:

Prior to clearing near wetland areas, flagging, staking, or fencing of the clearing limits shall be installed along the clearing boundaries in a clearly visible manner to ensure that no incidental disturbance occurs outside the permitted boundaries.  Prior to commencement of ground-disturbing work adjacent to wetlands or creeks, a more conspicuous, semi-permanent barrier fence, e.g. orange snow fencing attached to posts that also could serve to support the silt fencing, shall be installed along the clearing limits approved by this permit in order to prevent construction-related physical disturbances, e.g., the operation of equipment, placement of temporary stockpiles, clearing, etc., outside the footprint of the area to be developed.  The fencing shall be installed immediately after the clearing, but before ground disturbance activities take place.  The fence shall be maintained in good working order throughout active construction, and shall be removed when construction is complete.

Fill material must be clean, free from petroleum products and toxic contaminants in toxic amounts.  All fill shall come from previously authorized sources.  Temporary storage of excavated materials within the project boundaries, and disposal of excavated materials off-site must be managed to prevent sediment from being carried into adjacent wetlands and waters, and to prevent leachate from degrading water quality.  No fill shall be temporarily stored or placed in wetland areas.