# Central Region Survey Control Diagrams / Sheets

# Standards and Specifications

Function:  The Survey Control Sheet (SCS) identifies all existing horizontal and vertical survey control in relationship to the current project centerline or baseline.  This differs from a Survey Control Diagram (SCD) in that the SCD does NOT show the current project centerline/baseline.  The principal users of the SCS will be other Land Surveyors who are performing construction stakeout and boundary work in the vicinity of the project.  The SCS has the potential to be used for many years.  It's first user will typically be the surveyor staking out the current project.  Other near-term users are Land Surveyors staking the project centerline after construction, replacing corners that were disturbed, DOT surveyors checking that work, and the Project Engineer to ensure that existing monumentation does not get disturbed.

Please use the following 3 examples as a guide.

1)	Survey Control Diagram – No project centerline shown, no station & offset tables. Diagram is recorded as a Record of Survey.

2)	Survey Control Sheet, where ROW Mapping has been done – Project centerline shown, found monumentation not listed in tables. Sheet is used prepared to be used in the Departments plan set.

3)	Survey Control Sheet, where ROW Mapping has not been done – Project centerline shown, along with found monumentation station & offset tables. Sheet is used prepared to be used in the Departments plan set.

Sealed, Signed, and Recorded:  The SCS shall be sealed and signed by a Professional Land Surveyor licensed to practice in the State of Alaska.  The SCD will be filed as a Record of Survey (ROS).  A Surveyors Certificate shall appear and be signed by the PLS responsible for the diagram(s). The following certificate is an example:

Surveyor's Certificate

I hereby certify that I am properly Registered and Licensed to practice Land Surveying in

the State of Alaska, and that this drawing represents a survey made by me or under my direct supervision, and that the monuments shown hereon actually exist as described, and

that all dimensions and other details are correct to the extent shown hereon.

________________      ____________________________        ______________

(Surveyor Name)           (Surveyor Registration Number)            (Date)

Standards:  In the preparation of a SCD/SCS, adherence to the State of Alaska DOT drafting manual will be strictly enforced.  Read this document thoroughly since documents that do not substantially conform to these criteria will be rejected.

Scale:  The sheets shall be drawn at a scale to clearly show the relationship between survey control and the project centerline.  When selecting the scale, it must be remembered that all plan sets are published on 11”x17” sheets.

Content:

1.  Horizontal and vertical control points and existing corners shall be identified with standard DOT symbols.

2. Each SCS shall show basic topographic background using a screened pen.  The background will typically consist of the existing edge of pavement, buildings, significant land and water bodies.  Label side street names.  The goal here is to help orient the SCS user without cluttering the sheet.

3.  The current project alignment/baseline(s) shall be shown.  Tangents shall be labeled with bearings to the nearest second and distances to the hundredth of a foot.  Curves shall have PC and PT's stations and coordinates labeled as well as Delta, Radius, Length, and Tangent values.  On projects with multiple adjoining alignment/baseline(s) (such as ramp, side street, bike path, and frontage road alignment/baselines) the consultant shall meet with DOT Survey personnel to determine which alignment/baseline(s) shall be shown.

4. Cap marking details shall be drawn on the SCS as recovered or set.  The cap drawings shall depict all markings on the cap including dings and scratches as well as showing their orientation relative to north.

5.  Data Tables

A) Survey Control Table (SCT) - A table containing existing horizontal control points of sufficient quality to control the project.  These points can be set or found points.  The points shall be part of a closed traverse or redundantly tied points (as spelled out in the Statement of Services).  The SCT shall show point number; Stations and offsets referenced from the current project alignment/baseline(s), Northings and Eastings (to four decimal places), and a description of the type of monument shall be shown.  An example is shown below.

| Survey Control Points |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Pt. # | Station | Offset | Northing | Easting | Description |
| 2 | 54+24.62 | 17.98 Rt. | 145382.2883 | 432702.2884 | SET RBR/PC--PW-2 |
| 3 | 65+03.81 | 17.97 Lt. | 146243.9907 | 433354.2957 | SET RBR/PC--PW-3 |
| 4 | 85+89.35 | 18.10 Lt. | 147112.8755 | 435252.7555 | SET RBR/PC--PW-4 |
| 5 | 106+38.07 | 17.28 Rt. | 147135.7433 | 437302.5763 | SET RBR/PC--PW-5 |
| 18 | 237+55.29 | 25.37 Rt. | 148503.7821 | 450037.0610 | SET RBR/PC--HY-18 |

On projects with multiple sheets, the tables shall show only data applicable to the sheet on which the table is located.

- Vertical Control Table (VCT) - A table containing existing vertical control points of sufficient quality to control the project.   These points can be set or found points.  All vertical control points shall be part of a closed level loop; side-shots are not acceptable.  The VCT shall show point number, station and offset to the nearest foot referenced from the current project alignment/baseline(s), Northing and Easting to the nearest foot, elevations to the hundredth of a foot, and a description of Benchmarks and TBMs shall be shown.  An example is shown below.

| Vertical Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Pt. # | Station | Offset | Northing | Easting | Elevation | Description |
| 99 | 61+71 | 69 Rt | 145951 | 433192 | 324.91 | TBM-2: RR spike in 10” birch |
| 98 | 73+33 | 85 Lt | 146525 | 434140 | 327.21 | TBM-3[Old]: RR spike in 10” birch |
| 97 | 87+57 | 82 Rt | 147037 | 435433 | 348.15 | BM PW-1: Copper coated rod |
| 96 | 109+94 | 75 Rt | 147077 | 437657 | 348.62 | TBM-3: RR spike in 8” birch |
| 95 | 128+97 | 33 Rt | 147133 | 439560 | 373.91 | BM PW-2: NE base bolt signal/lum. Pole |
| 94 | 157+48 | 52 Lt | 147106 | 442411 | 387.00 | TBM-4: Copper coated rod |
| 93 | 177+82 | 43 Rt | 147136 | 444446 | 376.89 | BM PW-3: Copper coated rod |
| 92 | 201+87 | 108 Rt | 147106 | 446860 | 370.61 | TBM-5: RR spike in 10” birch |
| 91 | 219+98 | 79 Lt | 147375 | 448671 | 323.95 | TBM-6: RR spike 6” cottonwood |
| 89 | 236+25 | 78 Rt | 148379 | 449977 | 346.56 | BM PW-4: Copper coated rod |
| 88 | 236+29 | 82 Rt | 148373 | 449980 | 349.00 | TBM Hyer-6: RR spike in 18” birch |

On projects with multiple sheets, the tables shall show only data applicable to the sheet on which the table is located.

C) Additional Control - A table containing information about existing survey control that is not included in the other tables.  The points shown here do not meet the standards for being shown in the above listed tables.  Do NOT create this table or show these points if there is adequate control to stake the project shown in the Survey Control Table.  This table is a "last resort" source of control.  This table shall not have station and offset data for these points.  Show coordinates to the precision commensurate with the points shown. A note explaining the limitations/potential errors of these points shall accompany the Additional Control Table.

6.	Control Statements:

A Horizontal & Vertical control statement is required and will be provided by the Department of Transportation Survey Section.  An example is shown.

# Horizontal Control Statement

Coordinate System:

This project is located entirely within the Anchorage Bowl 2000 adjustment, a U.S. Survey Foot local surface grid coordinate system developed by the Alaska Department of Transportation.

Basis of Coordinates:

The Basis of Coordinates is NGS Station O’Malley, located near the intersection of the New Seward Highway and O’Malley Road. Said station has Anchorage Bowl 2000 coordinates of 303,939.2310 N, 353,362.5446 E.

Basis of Bearings:

The Basis of Bearings is a local plane bearing between NGS Station O’Malley and NGS Station Loop 2 USE RM 3 1964. NGS Station Loop 2 USE RM 3 1964 bears N 01º43’26. 4”E a distance of 49,488.45 U.S. Survey Feet from NGS Station O’Malley. NGS Station Loop 2 USE RM 3 1964 has Anchorage Bowl 2000 coordinates of 353,405.2778 N, 354,851.3982 E.

Translation Parameters:

To convert the local coordinates to NAD83 (92) State Plane foot coordinates, translate using +2,296,868.6878 N, +1,312,517.4905 E, and scale using 0.9998910192.

# Vertical Control Statement

MSL NAVD 88 as determined by differential level loops performed by AKDOT between bench marks USC&GS BM V-102 1965, a brass disk clamped to a copper coated rod, with an elevation of 356.05 feet, USC&GS BM D-103 1965, a brass disk clamped to a copper coated rod, with an elevation of 301.58 feet, and USC&GS BM E-103 1965 a brass disk clamped to a copper coated rod, with an elevation of 285.89 feet.

7. SCS Notes:

The following note shall appear on all sheets:

Whether listed or not, all monuments or property markers corners, or accessories which will be disturbed or buried, shall be referenced and re-established in their original position (A.S. 19.10.260) and recorded (A.S. 34.65.040).

Additional notes shall be added as necessary to improve the function of the SCS.

8. Tied rectangular corners with connecting lines labeled with bearings to the nearest second and distances to the hundredth of a foot.  Show Northings and Eastings to four decimal places at two existing corners on each sheet.

Additional Content  -

ONLY when ROW plans or Airport Property Plan is NOT a part of the project:

1.  A Monument Summary Table (MST) shall be included as a part of the SCS.  If ROW plans are a part of the project, the MST will be a part of the ROW plan set.  The MST shall show two types of corners: 1) all found corners, 2) monuments to be set as a part of the project.  The found corners that have the potential to be disturbed by the project shall also be marked with the appropriate Section 642 work task(s) required to preserve their position.  The MST shall include point number, station and offset from the current project alignment/baseline(s), Northing, Easting, Description, and the appropriate subsections of Section 642 of the Highways Construction manual as shown below.

| ## Monument Summary Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pt # | Station | Offset | Northing | Easting | Description | 642(8) | 642(9) | 642(10) | Record of Monument Form |
| 603 | 81+15.15 | 0.12 Rt | 146943.5926 | 434808.3546 | Fd DOT BC/Bx: PC 186+10.50 | X | X | X | X |
| 614 | 89+52.32 | 0.15 Rt | 147133.6145 | 435616.7452 | Fd DOT BC/Bx: PT 213+54.59 | X | X | X | X |
| 615 | 121+95.44 | 0.03 Lt | 147170.8447 | 438859.6427 | Fd DOT BC/Bx: PC 319+94.26 | X | X | X | X |
| 627 | 129+39.30 | 0.18 Rt | 147167.1395 | 439603.4657 | Fd DOT BC/Bx: PT 344+35.10 | X | X | X | X |
| 636 | 129+49.27 | 17.77 Rt | 147149.3468 | 439613.0643 | Fd DOT BC/Bx: SEC 1¦6\12¦7 *T17N R1W¦1E |  |  |  |  |
| 652 | 13687.87 | 0.18 Rt | 147151.5511 | 440351.8764 | Fd DOT BC/Bx: PC 368+90.49 | X | X | X | X |
| 654 | 143+90.67 | 0.19 Rt | 147147.6113 | 441054.6332 | Fd DOT BC/Bx: PT 391+95.87 | X | X | X | X |
| 655 | 155+45.60 | 5.10 Lt | 147164.0456 | 442209.4638 | Fd Rebar A/C {LS-5368} |  |  |  |  |
| 662 | 181+85.10 | 0.20 Rt | 147184.2140 | 444848.8881 | Fd DOT BC/Bx: SEC 6¦5\7¦8 *T17N R1E | X | X | X | X |
| 663 | 198+20.52 | 0.19 Rt | 147202.8089 | 446484.2086 | Fd DOT BC/Bx: PC 570+08.86 | X | X | X | X |
| 667 | 205+23.89 | 0.17 Rt | 147246.6726 | 447185.9247 | Fd DOT BC/Bx: PT 593+17.42 | X | X | X | X |
| 669 | 216+65.86 | 0.16 Rt | 147375.9332 | 448320.5483 | Fd DOT BC/Bx: PC 630+63.58 | X | X | X | X |
| 670 | 226+04.65 | 0.09 Rt | 147754.4878 | 449163.4640 | Fd DOT BC/Bx: PT 661+43.86 | X | X | X | X |
| 671 | 246+12.73 | 0.08 Lt | 149095.0208 | 450658.5692 | Fd DOT BC/Bx: PI 727+31.99 | X | X | X | X |
| 682 | 278+63.57 | 0.01 Lt | 151248.3772 | 453093.9458 | Fd DOT BC/Bx: PC 833+95.08 | X | X | X | X |

2.	Monuments to be set as a part of the project shall also be shown graphically when ROW plans or Airport Property Plan are NOT a part of the project.

3.	Sufficient right-of-way boundary information to ensure that the project is being constructed within lands that the project sponsor (typically DOT/PF) controls.