MEMORANDUM           STATE OF ALASKA

Department of Transportation and Public Facilities

Division of Statewide Design and Engineering Services

Right of Way Engineering - Survey Section

| To: | Consultant Surveyors | Date: AKSAS: | ## October 24, 2005 N/A |
| --- | --- | --- | --- |
| From: | Marty Williams/Michael Miller Survey Section Central Region | Tel No: Subject: | 269-0558 ROW Descriptors |

This memo shows the correct usage of descriptors for a ROW survey.

Follow the use of upper and lower case as shown in the examples.  In the majority of cases the information to the left of the colon is standard. Attention must be paid to Set or Fd. The information in the brackets will be used to identify who or what agency set the recovered monument. Who will be identified by an LS number (i.e. 7222-S or in those rare cases with an E) or company name. When describing square corners attention will be paid to the punctuation as it is shown in the examples. Township and Range is an important part of the description and will be included. When describing subdivision corners include all information that you feel is pertinent. Condition of the monument need not be included in the description since this information is recorded in the field notes. Full subdivision names should be used, however do not include Sub Or Subd as part of the descriptor. Lot and block can be shortened as shown in the examples. This is only a small sampling of the possible choices, and not all circumstances are identical, so use your best judgment. Prior to importing your ROW ties into your LDD project have your ASCII file reviewed by the office staff. Be prepared to make edits as directed.

Primary Government Monument

Fd BC[BLM]: C1/4 S36 *T17N R4W

Fd BC[GLO]: S19|S20/S30|S29 *T18N R1W

Primary Centerline Monument

Set BC/Bx[DOT]: PC 10+75.04

Fd BC/Bx[DOT]: Glenn Highway PT 1653+45.90 Ahd/1653+43.17 Bk

Secondary Centerline Monument

Secondary centerline monuments set.

Set Rbr/AC[DOT]: PC 1723+29.50

Property Primary Monument

Primary Monuments other than BLM/GLO monuments. This would include an Aluminum monument set at a section corner, if a private surveyor set the monument.

Fd BC[232-S]: Ref Mon (30' South) S8/S17 *T17N R1E

Fd AM[6925-S]: SW 1/16 S8 *T17N R1E

Fd BC[617-S]: ASLS 83-91 C5/PT ROW

Property Misc Corner

Any property corner other than a primary.  Concrete ROW monuments are also included.

Fd Rbr: NE L9 Blk3 Colonial Park

Fd Rbr: S Tr C ROW / Sec Line Inx

Fd Rbr: ROW PC L1 Blk3 Colonial Park

Fd Conc ROW Mon: KGB PT 911+45.5 100’ Lt

Fd Brg Tree Tag: RP to ¼ S8|S9 *T17N R1W

Fd Rbr/AC[5368-S]: NW L2 Blk6 Wandering Acres

Found Control Station

Published NGS, USG&GS, USLM, etc, control stations.

Fd BC/ROD[NGS]: Station BIG LAKE GPS 1986

Fd BC[NGS]: Station PURIN 1949

GPS Control Station

GPS control points.

Fd BC/SSROD: AKDOT GPS-44 1989

Set Rbr/AC: OGH 5.7 AZ

Survey Traverse Station

Fd Rbr[USKH]: TS OG-1

Fd RR Spike: Inx Pal-Was/Glenn

Set PK Nail: OGA-22

Set Rbr/PC: TS-7R

Secondary Control Point

Set Spike: TS-484

Set Mag Nail: TS-487

Vertical Bench Mark

Vertical bench marks.  (Does not include TBM’s.)

Fd BC[GAAB]: GAAB –113 BC in Pipe at Inx. Hillside and O’Malley

Temporary Bench Mark

Temporary bench marks.

Set RR Spike:  TBM Hyer 12” Birch Inx. Hyer Rd/Palmer Wasilla Highway

N bolt FH:  Scribed X on N bolt FH in NE quad Dimond & C Street