State of Alaska

Department of Transportation

&

Public Facilities

Highway Design

Central Region

P.O. Box 196900

Anchorage, Alaska   99519-6900

2005

Highway Design Checklist

Recommended For Approval:				Approved:

_____________________     ___________ 			____________________   __________

Brent D. Foley, Chairman	      Date			Robert A. Campbell, P.E.	 Date

Design Checklist Committee					Preconstruction Engineer

DOT Highway Design 					DOT Design & Construction

DEPARTMENT OF TRANSPORTATION

&

PUBLIC FACILITIES

(Central Region)

# HIGHWAY DESIGN CHECKLIST

## JANUARY 2005

Use this checklist prior to the PS&E review to ensure the Final Plans, Specifications, and Estimate (PS&E) package is complete and formatted in accordance with DOT &PF policy and design procedures.

The Lead Designer will initial the “designed” column and ensure another designer familiar with project development and design standards conducts an independent review and has initialed the “checked” column.

The completed Highway PS&E Checklist will be included in the project file.  The Transmittal Memo to the Contracts Section will include a reference to indicate the Highway PS&E Checklist is “completed and on file”.

# REVIEWERS

| TITLE | PRINTED NAME | SIGNATURE | INITIALS | DATE |
| --- | --- | --- | --- | --- |

Below is shown an outline of items required to complete an acceptable set of Plans, Specifications (Special Provisions), and Engineer’s Cost Estimate (PS&E).  Fill in each blank.  If an item is not applicable to the project use “N/A”.

## PLAN SHEETS

# TITLE SHEET (‘A’ Plan Sheets)

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner & Lt. to Rt.): - Revision Block (Used for Addendums) - State (Alaska) - Federal / State Project Numbers - Year (of Advertising) - Sheet Number (A1 for example) - Total Number of Sheets - CDS Route Number/ CDS Mile Points (Show the information above, except the “Total Number of Sheets” and CDS Route No./ Mile Point, on all other plan sheets) | __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ |
| 2. | Key Map w/ Project Location (arrow and closest town) |  |  |
| 3. | Vicinity Map: - North Arrow (w/ Township, Range, & Meridian) - BOP (Beginning of Project) Station - EOP (End of Project) Station | __________ __________ __________ | __________ __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 4. | Project Name or Title has the following format (example): - Roadway Name & Type of Improvement (Seward Highway Surface Rehabilitation) - Phase, if applicable (Phase II) - Milepost (MP 90.3-97.0) - Physical Limits (Girdwood to Bird Point) - Federal/ State (Construction) Project Numbers (The Project Name should closely match the name on the Federal-Aid Agreement. If applicable, all project stationing should be developed and stationed south to north and west to east.) | __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ |
| 5. | Project Work Description: The work items shown are listed in the order of work performed (for example: Grading, Drainage, Paving, Signing & Striping, Illumination, Signalization, Landscaping & Enhancements) |  |  |
| 6. | Index of Sheets will be in the following Alpha-Numeric order: ‘A’ Title, Legend/ Symbols (A2), Sheet Layout Schematic, and Survey Control ‘B’ Typical Sections ‘C’ Estimate of Quantities/ Table of Estimating Factors ‘D’ Summary Tables ‘E’ Details (Regional & Project Specific) ‘F’ Plan & Profile for Mainline Plan & Profile for Approaches Plan & Profile for Pathways ‘G’ Intersection Grading Plans ‘H’ Traffic Sheets (Signing, Striping, Illumination, & Signalization) ‘J’ Traffic Control Plans ‘K’ Automated Traffic Recorder (ATR) & Weigh-In-Motion (WIM) ‘L’ Landscaping ‘M’ Retaining Walls ‘N’ Bridge Structures ‘P’ Unassigned ‘Q’ Unassigned ‘R’ Right-of-Way Maps ‘S’ Unassigned ‘T’ Unassigned ‘U’ Utilities ‘V’ Unassigned ‘W’ Unassigned ‘X’ Unassigned ‘Y’ Unassigned ‘Z’ Unassigned (Note: Sheet A2, Legend/ Symbols, should be obtained from DOT. Do not use the letters ‘I’ &’O’) | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 7. | All DOT Standard Drawings (w/ asterisk “as modified Herein”) are listed. (All modified DOT Drawings are shown in the ‘E’ Details Section.) If a Metric project, add the following note below the usual “The following DOT Standard Drawings apply to this Project: ”ALL DOT STANDARD DRAWINGS ARE IN U.S. CUSTOMARY UNITS.” (Required since Metric DOT Standard Drawings are being eliminated.) | __________ __________ | __________ __________ |
| 8. | Project Summary: - Roadway Names (Major) w/ widths & lengths - Bridges w/ widths & lengths | __________ __________ | __________ __________ |
| 9. | Design Designation: - Functional Class - Current Year ADT - Design Year ADT - Design Year DHV (%) - Design Speed - Directional Split (%) - Trucks (%) - Equivalent Single Axle Loads (ESALs) (All Design Designations shall be obtained directly from the DOT/PF Planning Highway Data Section) | __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ |
| 10. | Title Block (bottom right hand corner) will have the following information: Departmental Projects: - DOT Project Manager Stamp w/ Signature & Date - DOT Design Chief Stamp w/ Signature & Date - DOT Regional Preconstruction Engineer “Approval” Signature & Date - DOT Regional Director, Construction & Operations “Concur” Signature & Date - DOT Construction Project Manager Signature & Date (To be signed after project “Asbuilts” are completed) - DOT Engineer of Record (EOR) Stamp w/ Signature & Date on all other plan sheets Consultant Projects: - Consultant Engineer of Record (EOR) Stamp w/ Signature & Date - Consultant Project Manager Stamp w/ Signature & Date - DOT Regional Preconstruction Engineer “Approval” Signature & Date - DOT Regional Director, Construction & Operations “Concur” Signature & Date - DOT Construction Project Manager Signature & Date (To be signed after project “Asbuilts” are completed) - Consultant Engineer of Record (EOR) Stamp w/ Signature & Date on all other plan sheets | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 11. | If Consultant Design, show “PLANS DEVELOPED BY: (Name of Firm) in 0.140 Text Size just above the Signature Block in the lower right hand corner. (The name of the Consultant’s firm shall be located just under the Consultant Engineer’s Seal on all other sheets. Consultant’s Logos are not allowed on any plan sheet.) | __________ | __________ |

LEGEND/ SYMBOLS (‘A’ Plan Sheets)

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | Sheet A2 shows all required legend & symbols used within the plan set (This sheet will be supplied by DOT and shall be stamped & signed by the EOR.) |  |  |

GENERAL LAYOUT (‘A’ Plan Sheets)

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | Stick drawing(s) of Roadway Centerline alignment(s) |  |  |
| 5. | Sheet outlines with Sheet numbers superimposed on alignment |  |  |
| 6. | Bubble Legends are no longer required. (Work items involving Removals, Relocations, Reconstructions, Adjustments, and Removal of Structures & Obstructions shall be shown in the Summary Tables) |  |  |

SURVEY CONTROL (‘A’ Plan Sheets)

All Survey Control Sheets shall be checked and approved by the DOT Survey Manager or his/her representative, prior to final placement in the Plan Set.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Registered Surveyor’s Stamp w/ Signature & Date |  |  |
| 4. | North Arrow |  |  |
| 5. | Stick drawing(s) of Roadway Centerline alignment(s) |  |  |
| 6 | Survey alignment with coordinate schedule table (Northings and Eastings) depending on if it is Metric or U.S. Customary Units |  |  |
| 7. | Basis of Horizontal and Vertical Control |  |  |
| 8. | If a scale is necessary, it should be bar scale rather than an inch scale (not accurate if plan sheets are half-sized) |  |  |
| 9. | Section Line Ties |  |  |
| 10. | Section Corners |  |  |
| 11. | Items listed above are generic. (Contact the DOT Survey Manager for the actual items required for the Survey Control Sheets to meet DOT Survey Standards) |  |  |

TYPICAL SECTIONS (‘B’ Plan Sheets)

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | General Notes (top right hand corner). Shown on the first Typical Section Sheet. These notes should cover all typical sections for items of work such as: - Clearing & Grubbing Limits - Topsoil & Seeding Limits | __________ __________ | __________ __________ |
| 5. | Typical Section Titles: (Show only the Roadway or Street names. The word “Typical Section” is shown only in the Title Block in the bottom right hand corner) |  |  |
| 6. | If applicable, show the “Cut Section” left of centerline and the “Embankment Section” right of centerline (Reduce the number of Typical Sections as much as possible) |  |  |
| 7. | Station-to-Station where Typical Section applies |  |  |
| 8. | Show “Typical” roadway surface width (left half shows total half-width; right half details lane and shoulder widths) (Transition widths are generally not shown on typical sections) |  |  |
| 9. | Show roadway structural section material types and depths in order, from top to bottom (All materials should match the Specification description) |  |  |
| 10. | Embankment & Ditch Foreslope and Backslope Rate (2:1, 4:1, etc.) |  |  |
| 11. | Sidewalk width and structural section material layer depths. |  |  |
| 12. | Pathway width and structural section material layer depths. |  |  |
| 13. | Median (Raised or Flush) width and structural section material layer depths. |  |  |
| 14. | Show Right-of-Way limits left & right of centerline. |  |  |
| 15. | Surface Cross Slopes (%) and directional arrow on: - Roadway - Median - Sidewalk - Pathway | __________ __________ __________ __________ | __________ __________ __________ __________ |
| 16. | Curb and Gutter Types: - Mountable Curb - Expressway Curb (medians) - Depressed Curb (approach curb cuts) - Rolled (residential streets) | __________ __________ __________ __________ | __________ __________ __________ __________ |
| 17. | Roadway Centerline, Profile Grade Point, and Point of Rotation. |  |  |
| 18. | Pathway Centerline, Profile Grade Point, and Point of Rotation. (If separate alignment & profile grade). |  |  |
| 19. | Cut / Fill Slope Exception Table. |  |  |
| 20. | Clearing (and Grubbing) Limits. |  |  |
| 21. | Topsoil (depth) and Seeding limits. |  |  |
| 22. | Surcharge Section: - Typical Section w/ Surcharge Depth Table - Surcharge Begin & End Transition | __________ __________ | __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 23. | Muck Excavation Section: - Typical Section – Width & Material Types - Elevation View – Show 1¼:1 slope pay limits down from the Traveled Way Line to firm bottom - Profile View – Show Begin & End Muck Excavation Transitions (length & slope rate) | __________ __________ __________ | __________ __________ __________ |

ESTIMATE OF QUANTITIES (‘C’ Plan Sheets)

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | The column headers shall be in the following order (left to right): Item Number, Item Description, Pay Unit, and Totals. (Pay Item numbers can be shown on the left and right side of the Estimate of Quantities sheet on large projects, if applicable) |  |  |
| 5. | Check that all Item Numbers, Item Descriptions, Pay Units, & Totals match that shown in the Cost Estimate, Bid Schedule, Plans (Summary Tables), and Special Provisions |  |  |
| 6. | All Item Numbers are in order, starting w/ the smallest item number at the top |  |  |
| 7. | All high dollar items reflect the “Degree of Accuracy” to adequately cover the quantity represented |  |  |
| 8. | Show the following “Table of Estimating Factors” on the last Estimate of Quantities Sheet: - Weights for Asphalt Pavements (lb/ ft3) - Asphalt oils (% of type of Asphalt Pavement) - Asphalt Tack Coat (ton/ yd2) - Aggregate Base Course (lb/ ft3) - Borrows (lb/ ft3) - Water For Seeding (gallon/ 1000 ft2) (The Regional Materials Engineer should confirm the Estimating Factors) | __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ |
| 9. | Every Pay Item on the “Estimate of Quantities” sheet is supported either in the “Plan Summary Tables” or the “Design Quantity Computations” |  |  |

## SUMMARY TABLES (‘D’ Plan Sheets)

Note:  The ‘D’ Section should contain summary tables for all work items except those shown on the Traffic Sheets (Signing, Lighting System, & Signal System), Bridge Sheets (Concrete Volumes, Reinforcing Steel, Piles, etc.), and the Landscaping Sheets (Plants, Trees, Boulders, Trail Amenities, etc.)

It is preferable to group certain work items.  (For example: Culverts & End Sections or Guardrail & End Treatments)

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 4. | Quantity totals do not have be shown at the bottom of each summary table (If the Designer chooses to do this, then the final quantities should match that on the Estimate of Quantities Sheet) |  |  |
| 5. | Clearing (and/or) Grubbing Areas |  |  |
| 6. | Removal of Structures and Obstructions |  |  |
| 7. | Removal Items (Pavement, Sidewalk, Culvert Pipe, Manhole, Inlets, Curb & Gutter, Fence. Etc.) |  |  |
| 8. | Mail Box Installation (single or multiple) |  |  |
| 9. | Railing (Pedestrian) |  |  |
| 10. | Culverts & End Sections (See note above for grouping items) |  |  |
| 11. | Storm Drain Manholes (Not Sewer), Sewer Facilities, and Storm Drain Pipes (See note above for grouping items) |  |  |
| 12. | For a major earthwork project the “Earthwork Summary Table” should always be the first summary sheet shown and should be on an individual summary sheet. Show the Contractor what design assumptions were made: - Total Estimated Unclassified Excavation - Estimated Waste Material - Estimated Usable Excavation - Usable Excavation Shrinkage Factor - Estimated Rock Excavation - Rock Excavation Swell Factor - Estimated Borrow Weights - Estimated Crushed Aggregate Weights - “Embankment Equation” (Has the Designer determined the Gravel, Riprap, & Asphalt Materials Source? This will determine the availability of materials and haul costs.) | __________ __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ __________ |
| 13. | Sanitary Sewer Manholes (If not shown in the Utility Sheets) |  |  |
| 14. | Reconstruct Manholes |  |  |
| 15. | Adjust Manholes |  |  |
| 16. | Relocate Manholes |  |  |
| 17. | Inlet, Type ___ |  |  |
| 18. | Adjust Inlets |  |  |
| 19. | Findrains, French Drains, & Subdrains |  |  |
| 20. | Guardrail & End Treatments (See note above for grouping items) |  |  |
| 21. | Fence and Noise Barriers |  |  |
| 22. | Curb Ramps |  |  |
| 23. | Curb & Gutter (all types) |  |  |
| 24. | Railroad items |  |  |
| 25. | Water Line Installation (If not shown in the Utility Sheets) |  |  |
| 26. | Fire Hydrants Adjustments and Installations |  |  |
| 27. | Adjust Valve Boxes |  |  |
| 28. | Insulation Board (square yard only w/ R-Value specified on Plans) |  |  |
| 29. | Residential, Commercial, and Public Approaches |  |  |
| 30. | Survey Monuments and Cases (if not shown on ROW Sheets) |  |  |
| 31. | Rest Areas (Benches, Picnic Tables, Toilets, Bumper Curb, Interpretive Signing, etc.) Use Pay Items 622, not 650, which are reserved solely for DNR projects. |  |  |

DETAILS (‘E’ Plan Sheets)

HIGHWAY DETAILS

All “Highway Details”, whether they are details developed by the Designer or the commonly used “Regional Details”, should be project specific.  All other details used for Signing, Striping, Illumination, & Signalization, Traffic Control, Landscaping, and Utilities should be shown in the appropriate section of the Checklist.

All “Regional Details” can be found either on the “L” Drive (DOT Library Drive) under L:/Engineering/Regional Details or on the Internet under the Department of Natural Resources (DNR) Site name ftp.cadastral.info/dot-cadastral/Engineering_Design_Meetings.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | All Regional Details are to be used as “Base Sheets” (generic details) and some can be modified to be more project specific |  |  |

PLAN and PROFILE (P&P) (‘F’ Plan Sheets)

### PLAN VIEW

The following items should be shown on the Plan View of each Plan Sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | North Arrow |  |  |
| 5. | The minimum scale of the “full size” Plan View will be as follows: - 50:1 for Rural Projects - 20:1 for Urban Projects | __________ __________ | __________ __________ |
| 6. | Beginning of Project (BOP) Station |  |  |
| 7. | End of Project (EOP) Station |  |  |
| 8. | Centerline Stationing |  |  |
| 9. | Show Mainline Station Equations, if applicable. Label “AHD.” And “BK.” after the Stations |  |  |
| 10. | Centerline Bearings (Shown as 7 digits) |  |  |
| 11. | Edge of Pavement Tapers w/ Stations & Offset Distances |  |  |
| 12. | Any important existing topographical features (roadways, approaches, buildings, bridges, curb & gutter, any ROW encroachments) |  |  |
| 13. | Match Lines (with next/ previous Sheet and Station) |  |  |
| 14. | Mainline and cross-street roadway names |  |  |
| 15. | Right-of-Way (R/W) & Public Use Easement Lines |  |  |
| 16. | Temporary Construction Easement lines (this includes approach easements) |  |  |
| 17. | All Existing Overhead Utilities (station & elevation above crossing point @ centerline): - Electric - Telephone - Cable TV - Miscellaneous | __________ __________ __________ __________ | __________ __________ __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 18. | All Existing Underground Utilities: - Electric - Telephone - TV Cable - Fiber Optics - Water Lines - Sewer Lines - Storm Drain Lines - Natural Gas Lines - Petroleum Fuel Lines - Miscellaneous | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ |
| 19 | Curve Data: - PI Station & Coordinates - Degree of Curve - Radius - Length of Curve & PC/ PT Stations - Rate of Superelevation - Runout, Runoff, & % Runoff | __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ |
| 20. | Mainline/ Street Intersection and Public Approach Radius Point Table, if applicable |  |  |
| 21. | Cut Backslope (dashed line) and Fill Slope (dotted line) Limits |  |  |
| 22. | All new roadway features such as curb and gutter, medians, drainage improvements, and pedestrian amenities (sidewalks & pathways) |  |  |
| 23. | Guardrail Locations and End Treatments |  |  |
| 24. | New retaining wall structures |  |  |
| 25. | New Bridge Structures |  |  |
| 26. | Existing Stream & River Channels |  |  |
| 27 | Coastal Tide lines |  |  |
| 28. | Storm Drain Structures & Pipes w/ tables (large storm drain projects may use separate P&P sheets) |  |  |
| 29. | New culverts w/ drainage arrows |  |  |
| 30. | Special Ditches & Linear Ditch grading locations |  |  |
| 31. | Section Line Ties |  |  |
| 32. | Section Corners |  |  |
| 33. | Silt Fence w/ Flow Arrows |  |  |
| 34. | Straw Bales w/ Flow Arrows |  |  |

### PROFILE VIEW

The following items should be shown on the Profile View of each Plan Sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | Original Ground (OG) Line & Labeled |  |  |
| 5. | Profile Grade Line & Labeled |  |  |
| 6. | VPI Stations & Elevations and Vertical Curve Lengths (VPC & VPT shown as circles - not labeled) |  |  |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 7. | Station Labeling at 100 Foot (U.S. Customary Units) or 20 Meter (Metric) increments shown at bottom of Profile Grid and at the sheet match lines |  |  |
| 8. | The grid lines (horizontal & vertical) conform to drafting standards addressed in the DOT Drafting Manual |  |  |
| 9. | All Profile Stations & Elevations are properly located within the Grid |  |  |
| 10. | Muck Excavation limits & approximate depth (with begin/ end transition length & stations) |  |  |
| 11. | Water Channels (Streams & Rivers) |  |  |
| 12. | Tide Lines |  |  |
| 13. | Subsurface Water Elevations |  |  |
| 14. | Existing Culverts |  |  |
| 15. | New Culverts |  |  |
| 16. | Bridge (begin & end stations/ elevations) |  |  |
| 17. | All Existing Overhead Utilities (station & elevation above crossing point @ centerline): - Electric - Telephone - Cable TV - Miscellaneous | __________ __________ __________ __________ | __________ __________ __________ __________ |
| 18. | All Existing Underground Utilities: - Electric - Telephone - Cable TV - Fiber Optics - Water Lines - Sewer Lines - Storm Drain Lines - Natural Gas Lines - Petroleum Fuel Lines - Miscellaneous | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ |
| 19. | Existing Culverts |  |  |
| 20. | New Culverts |  |  |
| 21. | Bridge (begin & end stations/ elevations) |  |  |
| 22. | Storm Drain Structures & Pipes (w/ Tables) |  |  |
| 23. | Special Ditches w/ Station, Offset, Elevation (begin, end, & vertical angle points, and ditch grades) |  |  |
| 24. | Datum (elevations) |  |  |

APPROACH PLAN & PROFILE

The following items should be shown on each “Approach Plan & Profile” Plan Sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | North Arrow |  |  |
| 5. | Approach Horizontal & Vertical Alignments |  |  |
| 6. | Intersecting Centerline (C/L) Stations for Mainline & Approaches |  |  |
| 7. | Sight Distance Triangles distances have been calculated for each residential, commercial, and public approach and are listed |  |  |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 8. | Intersection Data: - Road / Street Names - C/L Stationing and Intersection Station Equations - Centerline Bearings - Curve Data - Radius Point Table w/ Size, Station, and Offset Distance - Pavement Widths | __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ |

PATHWAY PLAN & PROFILE

The following items should be shown on each “Pathway Plan & Profile” Plan Sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | North Arrow |  |  |
| 5. | Approach Horizontal & Vertical Alignments |  |  |
| 6. | Intersecting Centerline (C/L) Stations for Mainline & Approaches |  |  |
| 7. | Intersection Data: - C/L Stationing and Intersection Station Equations - Centerline Bearings - Curve Data - Radius Point Table w/ Size, Station, and Offset Distance - Pavement Widths | __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ |

INTERSECTION GRADING PLANS (‘G’ Plan Sheets)

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | North Arrow |  |  |
| 5. | Intersecting Centerline (C/L) Stations for Mainline & Approaches |  |  |
| 6. | Label Manholes & Inlets (Structure number should match P&P and Summary Tables) |  |  |
| 7. | Intersection Grading Data: - Street Names - Station at C/L for each appropriate interval - Elevations & offset distances right & left of C/L - Begin & End Radius stations & elevations - Elevation & offset at approximately 20’ intervals - Rotation points and pavement cross slopes - Elevation & offset at the Curb Flow Line - Show appropriate breaklines | __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ |

TRAFFIC SIGNING, STRIPING, ILLUMINATION, & SIGNALIZATION (‘H’ Plan Sheets)

The following items should be shown on each “Plan View” sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | North Arrow |  |  |
| 5. | Roadway Names |  |  |
| 6. | Centerline Stationing |  |  |
| 7. | Match Lines (with next/previous Station) |  |  |

###### SIGNING

The following items should be shown on each “Signing Plan View” sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Sign post number, station, centerline reference, type, legend, and direction sign faces agree with the sign summary |  |  |
| 2. | Signs mounted on signal mast arms agree with the signal sheets |  |  |
| 3. | Stationing of signs mounted on electroliers agrees with the electrolier summary |  |  |
| 4. | Show STOP or YIELD signs and D3-1s at all non-signalized public approaches |  |  |
| 5. | Show signs graphically (sign picture) |  |  |

###### STRIPING

The following items should be shown on each “Striping Plan View” sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Include Striping Notes: - Call out surface applied (Non-Inlaid) or inlaid striping if methyl methacrylate is being used |  |  |
| 2. | Label the width and color of each marking in every window |  |  |
| 3. | Station the beginning and ending of each marking |  |  |
| 4. | Station the beginning and ending of all tapers |  |  |
| 5. | Show all dimensions where lane, shoulder, or median widths change |  |  |
| 6. | Lane and shoulder widths agree with the pavement widths shown on the Plan & Profile and Typical Section Sheets |  |  |
| 7. | Run center and edge lines through local roads and residential and commercial approaches |  |  |
| 8. | Station edge line and centerline breaks at approaches other than local roads and residential and commercial approaches |  |  |
| 9. | Flexible and Rigid Delineators: - Include the starting and ending point of all sections to receive delineators. |  |  |
| 10. | Dual left turn guide stripes |  |  |
| 11. | Crosswalks at signalized intersections |  |  |
| 12. | ONLYs and Arrows in turn lanes |  |  |
| 13. | Arrows in two way left turn lanes: - 500’ separation between arrow |  |  |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 14. | Combination Thru/Turn Arrows at split phased intersections |  |  |
| 15. | Lane Drop Arrows |  |  |
| 16. | RR Xing Symbols |  |  |
| 17. | Bicycle Lane Markings |  |  |
| 18. | Stop Bars at unsignalized intersections |  |  |
| 19. | Yield Lines |  |  |
| 20. | Gores |  |  |
| 21. | Rumble Strips |  |  |
| 22. | Right-in/right-out striping |  |  |
| 23. | Approach markings for obstructions on the non-intersection ends of raised medians per DOT Standard Drawing T-20 |  |  |
| 24. | Stop bars at all public approaches that are shown on the Plan & Profile sheets |  |  |
| 25. | 100 ft lead-in and 50 ft lead-out solid stripes at each location where white skip stripes between multi-lane thru lanes approach a major intersection |  |  |
| 26. | Pavement marker locations in relation to striping |  |  |

###### SIGN SUMMARY

The following items should be shown on each “Sign Summary” sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Include signing notes |  |  |
| 2. | Post number, station, centerline reference, type, legend, and direction sign faces in the sign summary sheets agree with the Plans |  |  |
| 3. | Use the DOT Northern Region Standard Sign Summary format (See the example in the Traffic Regional Details on the “L” Drive (Library Drive) under Engineering\ Regional\Traffic |  |  |
| 4. | Signs 36” wide and wider and diamonds 36” wide and wider on a side should be framed. All smaller signs should be unframed. - 8” D3-1 and 8” D3-1D signs are always unframed. - 12” D3-1 and D3-1A signs are always framed (There are no longer any braced signs) | __________ __________ | __________ __________ |
| 5. | Stationing of signs mounted on electroliers agree with the electrolier summary |  |  |
| 6. | Signs mounted on signal mast arms agree with the signal sheets |  |  |
| 7. | All signs panels are 0.125 inches thick |  |  |
| 8. | Check the area calculations: - Double the sign area of all signs installed as two signs back to back, including the 12” D3-1 and D3-1A signs - Do not double the area of 8” D3-1 or 8” D3-1D signs | __________ __________ | __________ __________ |
| 9. | Include a note in the remarks column for signs behind barrier, referencing DOT Standard Drawing S-32. Signs shielded by guardrail that are greater than 75’ from the end post do not require breakaway bases |  |  |
| 10. | Include sign shop drawings as an Appendix in the specs for all signs of variable dimensions and special signs |  |  |

###### ILLUMINATION

The following items should be shown on each “Illumination Plan View” sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Pole Numbers and locations agree with the Electrolier Summary |  |  |
| 2. | Show lighting circuits with conduit size and cable |  |  |
| 3. | Show and label load centers |  |  |
| 4. | Include electrolier summary |  |  |
| 5. | Include Luminaire Performance Criteria |  |  |
| 6. | Include Load Center Summary on sheets where load centers are shown, if space is available |  |  |
| 7. | Include Illumination Notes |  |  |
| 8. | Circuits from different load centers do not share junction boxes |  |  |
| 9. | Show numbered illumination j-boxes |  |  |
| 10. | J-Box numbering for lighting systems: - If there are no extra j-boxes, i.e. only j-boxes next to each electrolier, then the electrolier and the j-box should have the same number - If there are extra j-boxes, the extras should be numbered with the starting point electrolier number and an alpha suffix. (E.G., between electroliers 5 and 6, the intermediate j-boxes would be numbered 5A, 5B, 5C, etc. Between the Load Center C and electrolier 21, the intermediate j-boxes would be numbered CA, CB, CC, etc.) | __________ __________ | __________ __________ |

###### SIGNAL SYSTEM

The following items should be shown on each “Signal System Plan View” sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | North Arrow |  |  |
| 2. | Roadway Names |  |  |
| 3. | Size and configuration of all signal heads |  |  |
| 4. | Load center at least 30’ from the controller cabinet |  |  |
| 5. | Controller cabinet with door facing away from intersection |  |  |
| 6. | Signal Poles at least 15’ from edge of traveled way (desirable) |  |  |
| 7. | Mast Arms extending to the center of the lane farthest from the pole that is serviced by the mast arm signal |  |  |
| 8. | Signal Heads shown as they will be installed in accordance with DOT Standard Drawing T-30 and numbered by phase |  |  |
| 9. | Pedestrian Signals |  |  |
| 10. | Pedestrian push buttons |  |  |
| 11. | Optical Detectors positioned for line of sight operation |  |  |
| 12. | Loop Detectors |  |  |
| 13. | Loop Reference Line |  |  |
| 14. | Junction Boxes |  |  |
| 15. | Six loops maximum per Type 1A junction box |  |  |
| 16. | Tie-ins to existing facilities |  |  |
| 17. | Phase diagram: - Show all allowed pedestrian movements by a dashed line - Show permissive left turn movements with a dashed line | __________ __________ | __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 18. | Existing Utilities and conflicting topographical features |  |  |
| 19. | Poles and j-boxes agree with the Foundation and J-Box Schedule |  |  |
| 20. | Number poles clockwise from the northeast quadrant |  |  |
| 21. | Number pedestrian gear correctly |  |  |
| 22. | Number signal heads by correct phase and location around the intersection |  |  |

WIRING DIAGRAM

The following items should be shown on each “Wiring Diagram” sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Poles, j-boxes, signal heads, loops, and pedestrian signals and pushbuttons, agree with the plan view |  |  |
| 2. | Show conduit sizes and types for each conduit run |  |  |
| 3. | Show cable numbers, sizes, and types in each conduit run |  |  |
| 4. | Provide separate conduits for lighting and signal cable |  |  |
| 5. | Detail tie-ins to existing facilities |  |  |
| 6. | 1-3” RMC and 1-2” RMC in signal pole foundations |  |  |
| 7. | The Main Street is crossed once with 1-3” spare, 1-2” conduit for illumination, and 1-3” conduit for each signal pole across the street from the controller cabinet |  |  |
| 8. | Run conduits from signal poles across the street from the controller cabinet directly into the controller cabinet |  |  |
| 9. | Illumination circuit bypasses the controller cabinet |  |  |
| 10. | The controller cabinet feeder is the only circuit between the load center and the controller cabinet |  |  |
| 11. | Signal cable to the signal head at the end of the mast arm is 7C#14 |  |  |

SIGNAL SYSTEM SUMMARY

### The following items should be shown on each “Signal System Summary” sheet.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Show profile views of poles: - Mast arm lengths - Pole heights - Mounting heights - Signal heads - Signs - Electrolier mast arm lengths - Lane designations | __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ |
| 2. | Include Controller Equipment |  |  |
| 3. | Include Controller Details |  |  |
| 4. | Signs mounted on signal mast arms agree with sign summary and signing sheets |  |  |
| 5. | Pole numbers, signal head locations, lane designations, electroliers agree with plan view |  |  |
| 6. | Show Amplifiers: - Loops assigned to amplifier channels - Loops match plan view | __________ __________ | __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 7. | Include Load Center Summary |  |  |

###### TRAFFIC CONTROL PLANS (‘J’ Plan Sheets)

All Traffic Control Plans shall be reviewed and approved by the DOT Traffic Control Engineer or his/her representative, prior to final placement in the Plan Set.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | North Arrow |  |  |
| 5. | Vicinity Schematic: - Special Signing with Traffic Control Notes - Label all Street Names | __________ __________ | __________ __________ |
| 6. | Permanent Construction Signing: - Proper signing at the BOP & EOP (“Regional Detail” is available) - All Side Streets have the required Advance Construction Warning signing - Alternate Route (Detours) informational signs - Permanent “Double Fine” in work zone sign as per DOT Standard Drawing C-04 - Add Changeable Message Board if deemed warranted by the Traffic Control Engineer | __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ |
| 7. | Insert the Regional Detail sheet “Traffic Control Devices For Roadside” |  |  |
| 8. | Detours: Short Term (Overnight or Weekends): - Temporary Signing, Cones, and Barricades - Define Alternate Routes Long Term (Duration of Construction): - Permanent Signing & Barricades - Construction Sequencing (Phasing Plan) - Typical Section w/ lane and work area dimensions - Plan & Profile for detour alignment - Sign locations of Hospitals - Sign and define alternate routes | __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ |
| 9. | Pedestrian Access: Include DOT Standard Drawing D-03, “Pedestrian Traffic Control” - Access to all businesses is maintained throughout all phases of construction - Alternate pedestrian routes are identified, if required - Show all required directional devices (Cones, barricades, safety fencing, etc.) to separate pedestrians from the Work Zone & Traffic Lanes - Cross check curb ramp plans, striping plans, and traffic plans to verify crosswalk is the same location for all three, and the curb ramps are within the crosswalks, and the detector loops are placed correctly with respect to the crosswalks. | __________ __________ __________ __________ | __________ __________ __________ __________ |

### AUTOMATED TRAFFIC RECORDER (ATR) / WEIGH-IN-MOTION (WIM) (“K” Plan Sheets)

All Automated Traffic Recorders (ATR) / Weigh-In-Motion (WIM) designs shall be reviewed and approved by the DOT Highway Data Manager or his/her representative, prior to final placement in the Plan Set.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | All Plan Sheets are stamped & signed by a “Electrical Engineer” |  |  |
| 4. | The DOT Planning Highway Data Section has prepared all required “Design Designations” for the project |  |  |
| 5. | All Traffic Counter/ WIM structures are also shown on the Mainline Plan & Profiles sheets |  |  |
| 6. | The ATR/ WIM Plans conform to the current DOT Planning Highway Data format (Contact the Data Manager for current examples of Plan Sheets) |  |  |

###### ATR/ WIM  SYSTEM COMPONENTS

The following Details and Items should be shown on the ATR / WIM Plan Sheets.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Tie-in to existing electrical & telephone facilities |  |  |
| 2. | Existing Utilities and conflicting topographical features |  |  |
| 3. | Loop Detectors, Axle Sensors, and Dimensions Layout |  |  |
| 4. | Junction Box(s) Detail and Layout |  |  |
| 5. | Traffic Cabinet, Poles, and Foundations |  |  |
| 6. | ATR/ WIM Electronic Equipment |  |  |
| 7. | Conduit sizes and types for each conduit run |  |  |
| 8. | Cable numbers, sizes, and types in each conduit run |  |  |
| 9. | Provide separate conduits for Loops, Power, and Sensor Cable |  |  |
| 10. | Load Center Detail |  |  |
| 11. | Wiring Diagram Layout of complete system |  |  |

### LANDSCAPING (‘L’ Plan Sheets)

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | The following list of generic items should be shown on each Landscape Plan Sheet: - Project Designation (top right hand corner) - Title Block w/ Project Title & Sheet Description - Landscape Architect’s Stamp w/ Signature & Date - North Arrow - Landscape Legend (identifying each type of shrub, tree, trail amenity, etc. shown on the individual sheet) - Show right-of-way lines - Existing Utilities - Curb lines - Medians - Roadway centerline & stationing w/ station match lines - Landscape sheets reference power requirements, if any, to electrical sheets |  |  |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 2. | Total quantities are summarized on the first sheet (L1) in the “Landscape Summary” and each quantity agrees w/ the Estimate of Quantities Sheet & the Cost Estimate: - Trees & Shrubs - Boulders - Topsoil & Seed - Vegetative Erosion Control - Pedestrian Area Paving - Landscape Structures - Decorative Fences & Railings - Benches, Trash Receptacles, Trail Signing, & Other Furnishings - Decorative Lighting (if not shown on the Illumination Sheets) | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ |
| 3. | Each Plan sheet shows the layout of each item and are labeled w/ individual quantities listed: - Trees & Shrubs - Boulders - Topsoil & Seed - Vegetative Erosion Control - Pedestrian Area Paving - Landscape Structures - Decorative Fences & Railings - Benches, Trash Receptacles, Trail Signing, & Other Furnishings - Decorative Lighting | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ |
| 4. | Installation details are shown at the back of the ‘L’ sheets for: - Trees & Shrubs (planting & staking detail) - Boulders - Vegetative Erosion Control - Irrigation Plan - Pedestrian Area Paving - Landscape Structures - Decorative Fences & Railings - Benches, Trash Receptacles, Trail Signing, & Other Furnishings - Decorative Lighting | __________ __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ __________ |
| 5. | All items are placed outside the clear zone: - Trees - Boulders - Landscape Structures - Decorative Fences & Railings - Benches, Trash Receptacles, Trail Signing, & Other Furnishings - Decorative Lighting - Load center (if installed under this project) | __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 6. | All items do not obstruct intersection or approach sight distance, signs, signals, other permanent traffic control devices, luminaries, or utilities: - Trees & Shrubs - Boulders - Vegetative Erosion Control - Landscape Structures - Decorative Fences & Railings - Benches, Trash Receptacles, Trail Signing, & Other Furnishings - Decorative Lighting (including load center) | __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ |
| 7. | Maintenance Agreement: A Maintenance Agreement for non-highway facilities has been signed |  |  |
| 8. | Adequate snow storage space between the Roadway and planting areas, pedestrian facilities, and other landscape items is provided on site |  |  |

### RETAINING WALLS (‘M’ Plan Sheets)

All Retaining Wall designs shall be reviewed and approved by the DOT Foundation Engineer (Statewide Materials) or his/her representative, prior to final placement in the Plan Set.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Retaining Wall Title Sheet, should show the following: - Project Designation (top right hand corner) - Title Block w/ EOR Stamp & Signature (Show on all wall sheets) - North Arrow - Show all required “Retaining Wall Notes” specifying any requirements for special construction techniques and wall materials - Vicinity Map showing the proposed wall location within the Roadway Project limits - The wall quantities, along w/ any excavation & backfill quantities | __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ |
| 2. | Retaining Wall Detail Sheets should show the following: - A side view, or Typical Section, of the type of structure to be constructed w/ the horizontal control point called out (this is usually at the top face of retaining wall) - Original ground line and cut slope, if required, w/ slope rate, behind the wall - The excavation limits behind the wall and below the footings - Reinforcing Steel Table, if applicable - Back of Wall drainage system - Right-of-Way Line shown outside wall area (This is not a Construction Easement since this is a permanent structure to be owned & maintained by the Department) - Details of wall appurtenances (depending on the type of structure) such as wall curb, top of wall concrete copings, handrail, noise barrier, wall drainage outlets, location, and configuration of signs and lighting, including conduit locations | __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 3. | Retaining Wall “Plan View” Sheets should show the following: - North Arrow - Horizontal wall alignment, including the Begin and End Wall Stations and offsets from Roadway Centerline, along w/ any wall angle points - Original Ground Contour elevations (obtained by cross sections taken at 10’, 25’, 50’, or 100’ intervals depending on the size and length of the retaining wall) - Testhole locations w/ numbers (actual testhole soil information is usually shown after the Wall P&P Sheets) - New roadway with alignment, curb & gutter, & sidewalks - The relative location of new and existing utilities within the proximity of the retaining wall - Right-of-Way lines and Construction Easement lines | __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ |
| 4. | Retaining Wall “Profile View” Sheets: - Original ground line and proposed profiles in front of and behind the retaining wall - Elevation on top of wall at the beginning and end of wall, all profile grade break points, and roadway profile data at the face of wall - Elevations along the bottom of the wall footing - Elevation of highest permissible level of foundation construction (the top of the footings should be placed at least 2 feet below the frost line), location, depth, and extent of any unsuitable material to be removed and replaced - Elevation high water and normal water levels at stream locations, if applicable | __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ |
| 5. | Testhole Data Sheets: Show Testhole Numbers and soil gradation information. Do not include Design Recommendations. (This information can be shown on the Plan Sheets or given to the Bidders as a separate document at time of advertising) | __________ | __________ |
| 6. | Construction Sequencing Sheets: Show all required traffic control, access, and staged wall construction sequencing |  |  |

BRIDGE STRUCTURES (‘N’ Plan Sheets)

The Designer should use this checklist in development of the drainage structures (bridges, large culverts, drainage basins, etc).  If DOT Bridge Design develops the structure design then they should fill in the “Designed By” & “Checked By”.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | All Right-of-Way limits are shown in the General Layout and Site Plan sheets |  |  |
| 5. | There are not any conflicts between the new bridge location and any modifications to the new stream channel |  |  |
| 6. | Are all structures located within the ROW? If not, has the ROW Section been notified? |  |  |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 7. | The Designer should “double check” that the bridge structure: - Is constructed in the correct location - Has the correct deck cross slope and/ or super elevation - Bridge and Pathway widths match that shown on the P&P Sheets | __________ __________ __________ | __________ __________ __________ |
| 8. | Check Plan View against the roadway plans for each bridge structure: Bridge №:________, begin_________, end_________ stations Bridge №:________, begin_________, end_________ stations Bridge №:________, begin_________, end_________ stations | __________ __________ __________ | __________ __________ __________ |
| 9. | The Hydraulic and Hydrologic Summary Tables match the Final Hydraulic and Hydrologic Report |  |  |
| 10. | The Foundation Summary Table matches the Final Foundation Engineering Report |  |  |
| 11. | Check profile grade against the roadway plans for each bridge structure: Bridge №:________, begin _________, end_______ elevations Bridge №:________, begin _________, end_______ elevations Bridge №:________, begin _________, end_______ elevations | __________ __________ __________ | __________ __________ __________ |
| 12. | All bridge names conform to Bridge Design Memo # 1-15 (See the “Bridge Section’ of the “Highway Design-Aids” for Memo # 1-15) |  |  |
| 13. | All bridge components are addressed in the Specs for material requirements, const. tolerances, method of payment, etc |  |  |
| 14. | The bridge plan sheets have the following Plan Sheets: - General Layout (see Bridge Design Memo # 6-01) - Site Plan (see Bridge Design Memo # 6-02) - MSE / Retaining Wall / Foundation Layout - Abutments - Abutment Details Piers - Pier Details - Typical Section - Framing Plan - General Layout (see Bridge Design Memo # 6-01) - Site Plan (see Bridge Design Memo # 6-02) - MSE / Retaining Wall / Foundation Layout - Abutments - Abutment Details Piers - Pier Details - Typical Section - Framing Plan - Girders - Girder Details - Miscellaneous Details - Approach Slabs - Steel Bridge Railing - Log of Test Holes (See the “Bridge Section’ of the “Highway Design-Aids” for Memos # 6-01 & # 6-02) | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 15. | The bridge number is provided on the all bridge plan sheets |  |  |
| 16. | The vertical clearance under the bridge meets the current DOT Preconstruction Manual requirements |  |  |
| 17. | All Bridge Plans are stamped and signed by the Engineer of Record (EOR) |  |  |
| 18. | Plans are readable when reduced in size |  |  |
| 19. | All applicable standard plans have been identified |  |  |
| 20. | All specifications language are included in the specifications and NOT on the plan sheets |  |  |
| 21. | All required staged construction and traffic control has been shown |  |  |
| 22. | All cross references between plan sheets have been verified |  |  |
| 23. | The bridge railing to roadway guardrail transitions have been addressed |  |  |
| 24. | Provisions for current and future utilities (always inboard of the fascia girder) and blockouts in the diaphragms and backwalls are addressed |  |  |
| 25. | All bridge mounted utilities have been shown and properly identified on the plans |  |  |
| 26. | All reinforcing steel schedules / bend diagrams are provided on the bridge plans |  |  |
| 27. | Check for embankment treatment (matching of dirt to wall) at abutments and wingwalls |  |  |
| 28. | Check for side slope consistency or transition from roadway slopes (_______) to bridge side slopes at abutment (________) |  |  |
| 29. | The Plans, Specifications, and Estimate have been compared against one another and checked |  |  |
| 30. | All pay item names and numbers match the Specifications |  |  |
| 31. | The terminology used throughout the PS&E is used appropriately and consistently |  |  |
| 32. | The PS&E is free of brand names, product names, trade names, and other proprietary items |  |  |
| 33. | All estimated quantities appropriately rounded? |  |  |
| 34. | The “Length of Need” guardrail analysis has been performed for the guardrail at each end of bridge |  |  |
| 35. | The bronze Bridge Number Plates (one each end) shows the year the structure is to be constructed |  |  |

### RIGHT-OF-WAY MAPS (‘R’ Plan Sheets)

All Right-of-Way Maps shall be checked and approved by the DOT ROW Engineering Supervisor or his/her representative, prior to final placement in the Plan Set.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | Project Designation (top right hand corner) |  |  |
| 2. | Title Block w/ Project Title & Sheet Description |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date |  |  |
| 4. | The Right-of-Way plan set shall be in the following order: - Title Sheet, R1 - Legend Sheet, R2 - Tract Maps, R__ - Plan Sheets, R__ - Monument Summary Sheets, R__ | __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 5. | The total number of Right-of-Way (ROW) Sheets match the Sheet Index shown on the “Title Sheet”, Sheet A1 |  |  |
| 6. | All Pay Item Numbers, Descriptions, and Quantities shown on the Monument Summary Sheet match that shown on the Estimate of Quantities Sheet |  |  |
| 7. | The preferred scale for rural and urban Right-of-Way projects shall be 1’=100’ or 1”=50’ respectfully. (The Right-of-Way Engineering Supervisor will make the final decision on plan scale) |  |  |
| 8. | The Horizontal Alignments match the Plans |  |  |
| 9. | Station Equations shall be noted on the ROW Plan Sheets, “AHD” and “BK” shall be used |  |  |
| 10. | All cutslope and fillslope limits are clearly shown |  |  |
| 11. | All Primary & Secondary monuments requiring referencing prior to Construction are listed |  |  |
| 12. | The Primary & Secondary Monument & Monument Case quantities are listed |  |  |
| 13. | All Parcels acquired for additional Right-of-Way are shown |  |  |
| 14. | All Temporary Construction Easement (TCE) shown match that shown in the Plans |  |  |

### UTILITY SHEETS (‘U’ Plan Sheets)

All Utility Sheets shall be checked and approved by the DOT Utility Engineer or his/her representative, prior to final placement in the Plan Set.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | The number of Utility Sheets match the Sheet Index shown on the “Title Sheet”, Sheet A1 |  |  |
| 2. | In order, the first Utility Sheets should show all work that the project Contractor is required to do, any details needed, and quantity items required for completing the work. (This option is better than showing the Contractors work on the Plan & Profile ‘F’ Section because it reduces plan sheet clutter) |  |  |
| 3. | Engineer of Record (EOR) Stamp w/ Signature & Date on the project Contractor sheets |  |  |
| 4. | The remaining utility Sheets should show the work required to be completed by any of the Utility Companies (ML&P, ACS, Enstar, GCI, etc.) These sheets are usually full size copies of as those in the Utility Agreements. |  |  |

ADA COMPLICANCE (No Plan Sheet Letter Assigned)

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | All Sidewalks, Pathways, Trails, Curb Ramps, etc. meet current ADA Compliance Standards. (See the “ADA Compliance Section” of the Highway Design-Aids for specific requirements) |  |  |

### SPECIAL PROVISIONS

Below are listed a number of generic requirements in creating an acceptable Specification (Special Provision) Package.

It is the responsibility of the Engineer of Record (EOR) and the Specification Writer to work together to insure completeness of this portion of the Plans, Specifications, & Engineer’s Cost Estimate (PS&E).

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | The cover page of the Special Provisions shows the following (in order from top to bottom): - Standard Specifications for Highway Construction used (Metric or United States Customary) and date - Project Name and Federal/ State Construction Numbers (Replaces page footer w/ same information) - Engineers Stamp w/ Signature & Date (On DOT projects the DOT Specification Writer stamps, signs, & dates the Special Provisions. On Consultant projects the Engineer of Record (EOR) stamps, signs, & dates the Special Provisions.) | __________ __________ __________ | __________ __________ __________ |
| 2. | All Standard Pay Items modified, in any way, from the “Standard Specifications for Highway Construction” (current edition) are addressed in the Special Provisions |  |  |
| 3. | All New Pay Items are addressed in the Special Provisions under the following: - Description - Construction Requirements - Method of Measurement - Basis of Payment (Other Subsections will also apply on a project specific basis) | __________ __________ __________ __________ | __________ __________ __________ __________ |
| 4. | All applicable Standard Modifications and Statewide & Regional Special Provisions are included |  |  |
| 5. | All specification recommendations by DOT Maintenance & Operations are addressed |  |  |
| 6. | All specification recommendations by DOT Regional & Statewide Materials are addressed |  |  |
| 7. | All specification recommendations by DOT Environmental are addressed (This includes all Environmental Permits and the Landscaping requirements) |  |  |
| 8. | All specification recommendations by the DOT Regional Hydrologist are addressed |  |  |
| 9. | All specification recommendations by DOT Bridge Design are addressed |  |  |
| 10. | All specification recommendations by DOT Highway Construction are addressed |  |  |
| 11. | All specification recommendations by DOT Traffic Safety are addressed |  |  |
| 12. | All specification recommendations by DOT Utilities are addressed |  |  |
| 13. | All specification recommendations by DOT Right-of-Way Section are addressed |  |  |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 14. | The following applicable “Appendices” are at the back of the Special Provisions: - Permits (Environmental Documents) - *Permits (ROW Parcel Acquisitions and Temporary Construction Easements (TCE), if applicable) - Erosion Sediment Control Plan (ESCP) - **Construction Surveying Requirements - **Utility Safety Requirements - Material Certification List (MCL) - Sign Details Note: * If there are more than Six (6) TCEs in the Contract do not put them in the appendices. Instead, work with ROW to create the “ROW Summary” as outlined in the 3-11-04 Quarterly Design Meeting. * * DOT will insert these at time of advertising. | __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ |

### ENGINEER’S COST ESTIMATE

The Engineer’s Cost Estimate is an important step in putting together the Design Package because the Bid Schedule in the Contract is derived directly from it.  Checking the Plans and Special Provisions will verify that all required Pay Items are listed within the Cost Estimate.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | All required work on the project is covered under the appropriate Pay Items |  |  |
| 2. | All new Pay Items that were created by the Designer are consistent with those used in the current edition of the DOT Bid Tabulation Summaries |  |  |
| 3. | All standard Pay Items that are obtained from the current edition of the Standard Specifications for Highway Construction are checked for correct Item Number, Item Description, and Pay Unit (Note: If the Cost Estimate is used to develop the Bid Schedule the Pay Units should be spelled out as shown in the Standard Specifications) | __________ | __________ |
| 4. | All new Pay Items are addressed in the Special Provisions under the Description, Method of Measurement, and Basis of Payment Subsections |  |  |
| 5. | All Pay Item Numbers are listed in order |  |  |
| 6. | All Unit Prices are consistent with current bid item prices. |  |  |
| 7. | The multiplication (Pay Unit x Unit Price) for each Pay Item is correct and the total dollar amount for the “Basic Bid” is correct |  |  |
| 8. | Subtract the cost of the 644 Pay Items (Field Office, Field Laboratory, and Transportation) “Exclusions” from the Basic Bid dollar amount |  |  |
| 9. | Add the Construction Engineering cost percentage to the Subtotal (Basic Bid – Exclusions) at the following rate: - 15% for Departmental (DOT, DNR, MOA) Personnel - 25% for Consultant Engineering Personnel | __________ __________ | __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 10. | Add the appropriate percentage for the DOT Indirect Cost Allocation Plan (ICAP) to the Subtotal (Basic Bid, Less 644 Items) dollar amount to obtain the Grand Total |  |  |

### FINAL PS&E TRANSMITTAL PACKAGE (MISCELLANEOUS)

The following is a list of items required to transmit an acceptable “Final PS&E Package” to the DOT Contracts Section for advertising the project.

| No. | DESCRIPTION | DESIGNED | CHECKED |
| --- | --- | --- | --- |
| 1. | The quantities shown on the Plans (Estimate of Quantities), in Specifications (Special Provisions), and the Engineer’s Cost Estimate & Bid Schedule have been checked for accuracy and consistency |  |  |
| 2. | Specify the “Completion Date” for when the work is to be completed. Make sure this date has been discussed w/ the DOT Highway Construction Engineer (Using Calendar Days to determine the time to construct the project is allowable only through the PS&E design stage and will not be accepted by the Contracts Section) | __________ | __________ |
| 3. | The number of Trainee/ Apprentice work hours have been determined and verified w/ the CRO |  |  |
| 4. | The DBE Goal Percentage have been requested from the Civil Rights Office (CRO) and forwarded to Contracts: - This should be done no more then 2 months prior to advertising the project or when the Cost Estimate (Basic Bid Amount) has increased or decreased at least 4 percent. - DOT Designers give the CRO the latest Cost Estimate (Final PS&E). - Consultant Designers do the same only the Cost Estimate is given to the Consultant Coordinator, who forwards it to the CRO. - The CRO then returns a “Disadvantaged Business Enterprise (DBE) Subcontractable Items List”, which is put in to the As- Advertised Contract. | ___________ ___________ ___________ ___________ | __________ __________ __________ __________ |
| 5. | Design Quantities Computation Package: - 81/2” x 11” or 11” x 17” loose sheet assembly (printed single sided), no dividers, not bound, and no binders - Show all calculations for Earthwork Excavation, Borrow Materials, Aggregate Base, ATB, Asphalt Concrete, etc - Do not insert the Engineer’s Cost Estimate - Do not show costs associated w/ individual items - Do not insert quantities that are already summarized in the Plan Summary Tables | __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ |
| No. | DESCRIPTION | DESIGNED | CHECKED |
| 6. | “Final PS&E Transmittal Memo” to DOT Contracts (Review Engineer) will consist of the following information: - Project name & Federal/ State Project Numbers - Description of Work - Advertising Period (Usually 3 weeks) - Contract “Completion Date” - “PS&E” Comments & Responses (Checked) - Engineer’s Cost Estimate (Signed Original) - Bid Schedule - PDA Phase 4 Construction Funding (if completed) - DBE Goal Percentage - Special Notice To Bidders (Outlines all information made available to the Contractor) - Special Provisions w/ Appendices - ROW Summary - Plans (Full Size Mylars) (Note: DNR is the only agency currently approved for submitting “original” stamped, signed & dated 11”x17” paper As-Advertised Plan Sets) - Quantities Computations - Cross Sections - Geotechnical Data (This should consist only of Testhole and Soils information and no design recommendations) - Additional people you want to receive the As-Advertised Package | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ | __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ __________ |
| 7. | Completion of the “Design Summary Memorandum” (if applicable) that details various design intents, commitments, and concerns that have developed since the Final Design Study Report (DSR) was approved. |  |  |
| 8. | Completion of the “Environmental Commitments Memo” (if applicable) from the Designer to the Highway Construction Engineer that addresses a brief description of the project, outlines permits and environmental documents, any changes to the Environmental Document since being signed, and the DOT Environmental Staff assigned to the Construction Phase. (Contact the DOT Regional Environmental Coordinator, or his/ her representative for current document format) | __________ | __________ |
| 9. | A Disk containing the “As-Advertised” Plans, Special Provisions, & Cost Estimate has been developed. (This information along w/ any Addendum changes should be submitted to the “As-Built” Manager at a later date for safe storage in the fireproof safe.) | __________ | __________ |
| 10. | All project design records must be retained a minimum of three years after FHWA pays the final voucher. Other records such as the project design data, original agreements and contracts, final reports, diaries, photographs, and as-built drawings must be permanently retained. (Refer to Chapter 4, Subsection 450.20 of the Highway Preconstruction Manual (HPM) for more specific information.) | __________ | __________ |