MEMORANDUM           STATE OF ALASKA

Department of Transportation and Public Facilities

Division of Statewide Design and Engineering Services

Locations

To:		Date:	February 28, 2003

AKSAS No:

Phone No:	269-0558

From: Michael Miller, PLS   	                     Subject:	SEW-2 Coordinate

Translation Parameters

Coordinate System:

This project is located entirely within SEW-2, a U.S. Survey Foot local surface grid coordinate system developed by the Alaska Department of Transportation. Seward 2 extends from Mile 36 of the Seward Highway to Ingram Creek.

Basis of Coordinates:

The Basis of Coordinates is NGS Station PF-118, a standard NGS disc crimped to the top of a stainless steel rod, located near the intersection of the Seward and Sterling Highways. Said station has SEW-2 coordinates of 91039.5589 N, 81567.5685 E.

Basis of Bearings:

The Basis of Bearings is a local plane bearing between NGS Station PF-118 and AKDOT GPS Monument G-1, a brass disc crimped to a stainless steel rod, at the intersection of the Seward Highway and the road to Johnson Pass trail. AKDOT GPS Monument G-1 bears N 34º10’19”E a distance of 86090.95 U.S. Survey Feet from NGS Station PF-118. AKDOT GPS Monument G-1 has SEW-2 coordinates of 162267.2922 N, 129923.1560.

Translation Parameters:

To convert the local coordinates to NAD83 (92) State Plane U.S. Survey Foot coordinates, translate using +2296870.1057 N, +1640621.2309 E, and scale using 0.99988347.

.