TIN Certification

King Salmon Airport Stages 1-3 - AKSAS #53417

I hereby certify that an independent ground survey was performed under my supervision to obtain data for and to check the reliability of the electronic Triangular Irregular Network (TIN) surface model KSA_OG_ALL STAGES as defined in the following files:

#### KSA-Stages 1-3	                             		12/19/2008 	9:07 AM

KSA-Stages 1-3.pnt				541	12/16/2008  	09:50 AM

KSA-Stages 1-3.sdf				1 	12/16/2008 	09:50 AM

KSA-Stages 1-3.tin				1,117	12/16/2008  	09:50 AM

KSA-Stages 1-3bnd.bin				885	12/15/2008  	03:21 PM

KSA-Stages 1-3flt.bin				722	12/18/2008  	03:40 PM

D59029F_KSA-Stages_1-3_Survey-Points.xls	1,900

These files are in LDD format and were produced by DOWL Engineers (DOWL HKM) from a ground survey dated July 2005 to October 2005, July 2007 to August 2007, and November 2008.

The surface KSA_Stages 1-3 is comprised of a combination of the 2005, the 2007, and the 2008 ground surveys performed by DOWL HKM.  The TIN is developed from measured data and meets 1-foot contour accuracy standards.

Areas from the 2005 survey preformed by DOWL that fell within the 2007 Runway Safety Improvement Project were not included in the KSA_Stages 1-3 surface.

The ground survey was performed by DOWL Engineers using a Leica TCRA1105 5-second robotic total station running TDS data collection software, a Leica TCRP+1205 5-second robotic total station using proprietary software, and also using a Real-Time Kinematic (RTK) GPS system composed of two Leica SR530 dual frequency receivers using one as a Base Station, and one as a Rover.

DOWL Engineers mapped approximately 249 acres of ground in the KSA_Stages 1-3 TIN.  The entire surface was visually inspected to verify accuracy of the contours.  The ground survey checked this TIN with over 155 surveyed points.  The measured coordinates of these points were compared to the contours as an independent check and were found to meet the specified accuracy as qualified above.  In addition, baseline station points were compared to ground shots taken near these points and found to meet the accuracy required during the quality control process.  Based upon these checks the KSA_Stages 1-3 TIN was determined to be capable of generating contours where 90 percent of the elevations tested would be within one half the specified contour interval, and that the remaining 10 percent of the elevations tested would be in error more than one half the specified contour interval (National Mapping Standards).

# Stanley E. Ponsness, PLS 6714

# DOWL Engineers