MEMORANDUM           STATE OF ALASKA

Department of Transportation and Public Facilities

Division of Statewide Design and Engineering Services

Right of Way Engineering - Survey Section

| To: | Mark Mayo Planning | Date: AKSAS: | ## June 1, 2004 57145 |
| --- | --- | --- | --- |
| From: | Michael Miller, P.L.S. #### Assistant Survey Manager Survey Section Central Region | Tel No: Subject: | 269-0558 Port Lions Professional Seals/ |

### As a follow up to our meeting with HDR and Lyle Riggins the following information was discussed.

- The Horizontal and Vertical control shall be CORS based with a NAVD 88 elevations established at the airport. The surveyor shall either recover or establish two permanent monuments for this project. These monuments shall be controlled with GPS for a minimum of six hours.  Using the NGS website a mean OPUS solution shall be provided to the Department for each point.

- Check shots to verify the TIN shall be taken with ½ of the shots taken provided to Kodiak Mapping and the remainder to be kept by the surveyor for an independent check. The length of the check shots shall not exceed 600 feet.

- Rights-of-Entry are required for Tracts 4 thru 6, 8 thru 11, and ADL 36998 as shown on BLM’s preliminary 14C survey. A copy of the 14C survey shall be provided to the Right-of-Way agent as soon as one is assigned. Please let us know which agent is assigned to your project.

- We will provide the surveyor the scaling and translation parameters to be used for this project once he provides us with the Latitude and Longitude he establishes for this project.

- From the time that the statement of services was prepared and you completed negotiations a couple of surveying items appear to have been overlooked. It is critical that this Department receives a Survey Control Diagram and survey report for this project. This information must be reviewed and accepted by the Survey Section before the development of a TIN is started. This is the underlying base for this project as it develops. Since a project such as this can cover a number of years we need this information in our records. Please direct HDR to have their surveyor sub prepare a report and survey control diagram to meet the Departments criteria. The report shall need to be in a three-ringed binder. There are numerous examples of a survey control diagram available to the surveyor at our FTP site.

If you have any questions, please feel free to contact me at 269-0558.

MM

Attachment: