MEMORANDUM           STATE OF ALASKA

Department of Transportation and Public Facilities

Division of Statewide Design and Engineering Services

Locations

To:		Date:	March 10, 2005

Phone No:	269-0558

From: Michael Miller, PLS   	                     Subject:	Mat-Su SV-1

Control Statement

Coordinate System:

This project is located entirely within the Matanuska-Susitna Valley SV-1 adjustment, a U.S. Foot local surface grid coordinate system developed by the Alaska Department of Transportation.

Basis of Coordinates:

The Basis of Coordinates is CGS Station S-1 (1922), a standard disk set in the top of an eleven-inch square monument flush with the ground, is located approximately 0.05 miles west of the Alaska Railroad station in Wasilla. Said station has SV-1 coordinates of 145,360.5554 N, 425,361.4974 E.

Basis of Bearings:

The Basis of Bearings is a local plane bearing between CGS Station S-1 (1922) and AKDOTPF Station GPS 44 (1989). AKDOTPF Station GPS 44 (1989), a brass disc clamped to the top of a stainless steel rod, which is located at approximate milepost 46.1 of the Parks Highway, bears N 87º 26’ 59.7” W a distance of 19969.57 U.S. Feet from CGS Station S-1. AKDOTPF Station GPS 44 (1989) has SV-1 coordinates of 146,249.0589 N, 405,411.7053 E.

Translation Parameters:

To convert the local coordinates to NAD83 (92) State Plane U.S. Foot coordinates, translate using +2,624,946.5706 Feet N, +1,312,508.8943 Feet E, and scale using 0.9998990458.

.