MEMORANDUM           STATE OF ALASKA

Department of Transportation and Public Facilities

Division of Statewide Design and Engineering Services

Locations

To:	 Jim Robar, PLS	Date:	June 12, 2007

Phone No:	269-0558

From: Michael Miller, PLS   	                     Subject:	Mat-Su SV-1

Control Statement

Coordinate System:

This project is located entirely within the Matanuska-Susitna Valley SV-1 adjustment, a U.S. Foot local surface grid coordinate system developed by the Alaska Department of Transportation.

Basis of Coordinates:

The Basis of Coordinates is the published positions of the following NGS CORS sites: TLKA Talkeetna CORS ARP N 62° 18’ 27.55614”   W 150° 25’ 12.97660”, TSEA Anchorage CORS ARP   N 61° 11’ 14.37495”   W 149° 53’ 41.81980”, and ZAN1   Anchorage WAAS 1 CORS ARP   N 61° 13’ 45.12939”   W 149° 46’ 48.80456”

Basis of Bearings:

The Basis of Bearings is a local grid bearing between BM Park (Point #50) and GPS FIRE (Point 54). These points were established by multiple static GPS observations.

BM Park is a 2.5” Brass Datum Disc on a 9/16” Stainless Steel rod, driven to refusal, set in a grassy knoll in the parking area of the Kepler/Bradley Lake Park, bears S 28° 21’ 52” W a distance of 40,640.37 U.S. Feet from Point 54, a 2” Aluminum Cap set on a 5/8” rebar in the paved parking lot of the Harold Stephan Fire Station.

Translation Parameters:

To convert the local coordinates to NAD83 (CORS EPOCH 2003?) State Plane U.S. Foot coordinates, translate using +2624946.5706 Feet N, +1312508.8943 Feet E, and scale using 0.9998990458.

To convert NAD83 (CORS EPOCH 2003.0000) State Plane U.S. Foot coordinates to local coordinates, scale using 1.00010096439 and translate using -2624946.5706 Feet N, -1312508.8943 Feet E.